NILM ML Technical Plan

Goal

Disaggregate whole-house aggregate power into per-appliance power estimates (and derived daily kWh / usage sessions) for a shortlist of priority appliances.

Phase 1 — Data & Signal Preparation

1) Define the production signal (what the device will output)

- Target LF NILM stream: Active Power (P) at 1–5 seconds sampling, 10 seconds max.

2) Create a training dataset that matches production

From the available dataset (HDF5):

- Extract or compute aggregate active power over 1–5 s windows.

- Align with ground-truth appliance channels.

- Standardize time indexing, handle missing points, and ensure consistent units (Watts).

3) Split data correctly

- Use time-based splits and/or building-based splits (no random row split).

- Keep a “future-like” test set that simulates real deployment (unseen periods / unseen buildings).

Deliverable: Clean LF dataset + reproducible preprocessing scripts.

Phase 2 — Modeling Approach

4) Start with one model per appliance

Instead of one unconstrained multi-output model:

- Train one sequence-to-point model per appliance.

- Input: aggregate window (e.g., last 300–900 seconds depending on sampling)

- Output: appliance power at center time-step.

5) Baselines first

Before Transformers:

- Baseline 0: always-off / mean predictor

- Baseline 1: Light CNN or LSTM seq-to-point
Only adopt Transformers if they beat the baseline on real watt/kWh metrics.

Phase 3 — Constraints & Post-processing (Make outputs physically valid)

6) Enforce physical plausibility

7) Convert power estimates into product features

- Daily/weekly kWh per appliance

- Usage sessions (ON/OFF via threshold + minimum duration rules)

- Confidence score (e.g., based on margin to threshold / model uncertainty)

Phase 4 — Evaluation (Report metrics that matter)

8) Appliance-level metrics (in real units)

Per appliance:

- MAE (Watts), RMSE (Watts)

- Daily energy error (kWh/day)

- ON/OFF detection F1 (thresholded)

Phase 5 — Deployment Readiness

9) Package the model for production use

- Fixed preprocessing (same scaling, same windowing)

- CPU-friendly inference (edge or cloud; target latency defined)

- Versioned model + reproducible training config

Critical flaws in the current approach (In report)

1. Resampling to 1-minute destroys appliance signatures

They state:

“Data was resampled to a 1-minute frequency”

Why this breaks NILM:

- Many appliances (fridge compressor, kettle, microwave, washer motor states) have on/off dynamics at seconds-scale

- At 1-minute resolution:

  - Transients disappear

  - ON/OFF edges smear

  - Multiple appliances overlap into a single averaged value

This is exactly why the fridge prediction plot is flat (~9–10 W) while the real fridge cycles at ~35–40 W.

➡️ The model is learning:

“The fridge is usually around its mean power”

Not:

“The fridge turned ON here”

2. Aggregate power ≈ 2 W while appliances are predicted at 10–25 W

Your bar charts show a physical impossibility:

- Aggregate input power ≈ 2 W

- Predicted appliances:

  - Dishwasher ≈ 25 W

  - Freezer ≈ 15 W

This violates energy conservation.

This means:

- The model is not constrained

- Outputs are independent regressions, not a valid disaggregation

- The model can “invent power”

This alone invalidates the qualitative claims in Section 3.2.

3. Multi-appliance output without energy or sparsity constraints

They predict:

“Predicted power consumption for multiple appliances at the current timestamp”

But:

- No sparsity (most appliances should be OFF most of the time)

- No ON/OFF state modeling

This guarantees over-prediction and appliance “bleeding”.

4. Metrics are reported on scaled values only

They report:

MAE ≈ 0.197 (Scaled)

Problems:

- Scaled MAE hides watt-level error

- No per-appliance MAE (W)

- No energy error (kWh/day)

- No ON/OFF F1 score

5. The qualitative claims are not supported by the plots

They claim:

“capturing cyclic patterns effectively”

But your fridge plot clearly shows:

- Ground truth: square-wave cycling

- Prediction: flat baseline

This is mean regression, not cycle tracking.