You can absolutely lead this as the “ML guy” — but you need to frame it correctly to the startup team: NILM is doable for a useful MVP, but accuracy depends heavily on what the hardware measures and what appliances you target.

Below is a step-by-step plan (what to do) and the “expert talking points” (what to tell them), grounded in the paper you shared.

First: important correction about your paper/dataset

The paper defines LF NILM as up to 1 Hz, but the LIT-Dataset they propose is actually high-frequency: it samples voltage + aggregated current at 15 kHz and includes precise event labels (~5 ms accuracy). (MDPI)

So if your startup says “we’re using LF NILM”, you must clarify one of these is true:

- They will downsample / compute low-frequency features from the 15 kHz recordings (totally possible), or

- They’re actually collecting LF data in the field (e.g., 1 Hz active power) and just using this paper as reference.

Either way, the plan below works — you just adjust the model/features.

What to tell the startup (your “expert opinion” in plain terms)

1) What NILM can and cannot do (for an MVP)

- Works best for a shortlist of “big, distinct” appliances (AC, fridge, kettle, water heater, iron, microwave).

- Struggles with many small electronics and similar-looking loads (chargers, TVs vs monitors), especially at low frequency.

- Overlapping usage (multiple appliances on) is the hard case; datasets like this include single and multiple concurrent loads specifically to test that. (MDPI)

2) Hardware requirements for ML success (non-negotiable)

Tell them: “Your ML performance ceiling is set by your sensing.”
Minimum recommended for LF-style NILM:

- Accurate timestamps (clock drift ruins labeling/training)

- Voltage + current (or at least real power + voltage), sampled consistently

- Good calibration and stable measurement noise profile
This paper emphasizes precise event recording/labeling and controlled switching because it matters a lot for learning signatures. (MDPI)

3) Product framing: “recommendations”, not perfect submetering

Position it as:

- “We estimate appliance usage well enough to generate actionable insights”
rather than:

- “We can perfectly identify every device always”

That’s the startup-friendly, realistic story.

What you should do (end-to-end ML execution plan)

Step 0 — Lock the target and success metric (MVP definition)

Do this before modeling:

- Pick 5–10 appliances to support in v1.

- Decide outputs:

  - Energy per appliance per day (kWh)

  - On/off events (usage sessions)

  - Real-time power estimate (optional)

- Define success metrics:

  - Energy error (e.g., MAE in watts, daily kWh error)

  - Event detection F1 (if you care about on/off)

Step 1 — Data audit and alignment with your hardware

From the paper, the dataset is organized in subsets (Synthetic/Simulated/Natural). (MDPI)
For your startup, map that to:

- Training data: controlled recordings (like their Synthetic idea)

- Hardening data: real homes (Natural)

- Edge cases: simulated or stress scenarios (optional)

Key decision:

- If your device measures LF (1 Hz power) → train models on low-frequency features.

- If it measures waveforms → you can still deploy LF-style outputs, but training can leverage richer signals.

Step 2 — Build the dataset in the format your pipeline needs

Use NILM tooling where it helps. The paper explicitly mentions NILMTK and its data format expectations. (MDPI)
Practical approach:

- Standardize to: timestamp, mains_power, (optional) voltage, (optional) reactive_power

- Ground truth for training (in lab): per-appliance power or at least event logs.

Step 3 — Choose your modeling approach (pick 1 for MVP)

You have two common LF NILM strategies:

A) Event-based (simpler, good for MVP if appliances have clear edges)

- Detect “events” (power changes)

- Classify which appliance caused each event

- Reconstruct usage sessions

B) Sequence-to-point / sequence-to-sequence regression (more robust)

- Input: a window of aggregate power (e.g., last N seconds)

- Output: estimated power of one appliance (train one model per appliance)

For a startup MVP, I’d recommend:

- Sequence-to-point per appliance for the top devices (often performs well and is easier to productize)

- Add event detection later for cleaner “usage sessions”

Step 4 — Baselines first (to avoid getting stuck)

Before deep learning, implement:

- A dumb baseline: “always off” / “mean power”

- A simple model: RandomForest/XGBoost on window features
This tells you if the signal supports your target appliances at all.

Step 5 — Train + evaluate properly (avoid fake accuracy)

Critical: split data by house / time, not random rows.

- If you random-split time-series, you get inflated results.
You want to know: “Does this generalize to a new home?” (startup reality)

Step 6 — Deal with “new home” adaptation (startup reality #1)

NILM models often degrade in new houses due to:

- different appliances

- different voltages

- different user habits

So bake in one of these:

- Calibration week: model learns the home baseline

- Light onboarding labels: user confirms a few detected events (“Was that kettle?”)
Active learning / light fine-tuning is common in LF NILM practice.

Step 7 — Turn predictions into recommendations (the product value)

Your system should produce outputs like:

- “AC usage is 35% higher than your last 2 weeks baseline”

- “Fridge cycles are unusually frequent → possible seal issue”

- “High nighttime base load → devices left on standby”

- “Top 3 energy drivers today: AC, water heater, iron”

This is where you become “startup useful” even if disaggregation isn’t perfect.

Step 8 — Deployment architecture (edge vs cloud)

Decide early:

- Edge inference (cheap model, privacy, low latency)

- Cloud inference (heavier model, easier iteration)

For LF NILM, edge inference is often feasible.

Step 9 — Monitoring and continuous improvement (startup reality #2)

In production you need:

- data drift checks (changes in appliance set)

- confidence scores (“high confidence kettle event”)

- feedback loop (user corrections → model improvements)

What I would present to them as your “ML plan” (one slide worth)

- Scope v1: 5–10 major appliances, daily kWh + usage sessions

- Data plan: lab ground truth + pilot homes

- Model plan: per-appliance seq-to-point regression + confidence scoring

- Reality check: best on large distinct appliances; small electronics limited at LF

- Roadmap: onboarding calibration + personalization + expanded appliance set

One more thing: dataset access

The paper states the LIT-Dataset is publicly available upon free registration and gives the dataset site. (MDPI)

If you tell me exactly what your hardware records (e.g., P only @1Hz, or V+I waveform, or P+Q+V), I’ll pin this down into:

- the exact feature set,

- model choice,

- training labels you must collect in your lab,

- and what appliance list makes sense for Ghana homes (realistically).

You’ve got two clean ways to handle this, and which one you choose should be driven by what the device will output in production.

The key principle

Train on data that matches what you’ll have at inference time.
If the startup will ship a device that samples at ~1 Hz (LF NILM), then your model should ultimately be trained/evaluated on 1 Hz-like features, even if the dataset is 15 kHz waveforms.

The LIT dataset in that paper is waveform-level (15 kHz V & I).
So you’ll need a feature/sampling strategy to bridge dataset → device.

Option A (recommended if your device is LF): “Downsample + compute LF features”

You treat the dataset as “raw sensor data” and derive LF meter-like signals.

What you do

- From 15 kHz voltage/current, compute power signals over windows:

- Active power (P) (mean real power)

- Reactive power (Q) (optional but useful)

- Apparent power (S), RMS current/voltage, power factor (PF)

- Choose a target sampling rate that matches device plans:

- 1 Hz (one value per second) is typical LF NILM

- 0.2 Hz (one per 5 sec) sometimes used for lower bandwidth

- Train NILM models using those LF features.

Why this is good

- Your ML becomes directly portable to the device later.

- You avoid building a “waveform model” that can’t run on your hardware.

What to tell them

- “We will use the dataset’s high-rate waveforms to simulate smart-meter/LF readings and train the disaggregation model for the same sampling rate the device will produce.”

Option B (if your device will actually measure waveforms): “Train HF model now”

If the hardware will sample kHz and you can afford it (storage + compute), you can train a waveform-based model now.

But be careful

HF models can look amazing on paper/dataset and then fail in real homes unless:

- the sensor chain (ADC, filtering, noise) is similar

- voltage waveform quality is stable

- you handle domain shift well

What to tell them

- “HF can be higher accuracy but increases hardware cost and deployment complexity; if we ship LF sensors, we should not train an HF-only model.”

The practical decision guide (use this in your meeting)

Ask/confirm internally (you don’t need to ask me):

- Will the device store/transmit waveforms (kHz) or only power summaries (1 Hz)?

- What’s the MCU/edge compute capacity?

- Do you care about small electronics, or only big loads?

Then choose:

If device outputs P (and maybe Q) at ~1 Hz → choose Option A.

This is the most common and startup-friendly.

If device outputs waveform data → Option B is possible, but you still may export LF outputs for product features.

How to go about it step-by-step (what you should do now)

Step 1 — Build two datasets from the same raw files

From LIT waveforms:

- LF dataset: 1 Hz P (and Q/PF if possible)

- HF dataset (optional): short waveform segments around events

This way you can:

- deliver an LF MVP model

- still keep a path for HF improvements later

Step 2 — Pick a realistic NILM target list (MVP)

Start with 5–8 appliances that are easiest at LF:

- kettle / water heater

- iron

- fridge

- AC / fan (depending on data)

- microwave / cooker
These give you the strongest chance of success.

Step 3 — Train the LF NILM model (MVP)

Best MVP approach:

- Per-appliance seq-to-point regression (one model per appliance)
Input: aggregate power window (e.g., last 60–300 seconds)
Output: appliance power at the center timestamp

Evaluate with:

- MAE (watts)

- daily energy error (kWh)

- optional event F1 (if you post-process into ON/OFF)

Step 4 — Plan for real-world testing (domain shift)

When the device arrives:

- record aggregate + a short period of ground truth using smart plugs for a few appliances (even 1–2 weeks)

- fine-tune or calibrate thresholds per home

What you should tell them about the “sampling mismatch” (copy/paste-ready)

- “The dataset uses 15 kHz voltage/current waveforms. Our production device is planned for LF NILM, so we will compute meter-like features (active power, reactive power, RMS, power factor) and downsample to 1 Hz to match the device output. That way the model we build now will transfer directly to field data. If later we decide to collect waveforms in the device, we can train an HF model as an upgrade.”

This is the “expert” answer.